this is gnb.sa.band78.fr1.51PRB.usrpb210.conf file---------------------

---------------------------------------------------------------------------------------------------------------------------------------------

Active_gNBs = ( "gNB-OAI");

# Asn1_verbosity, choice in: none, info, annoying

Asn1_verbosity = "none";

gNBs =

(

{

////////// Identification parameters:

gNB_ID    =  0xe00;

gNB_name  =  "gNB-OAI";

// Tracking area code, 0x0000 and 0xfffe are reserved values

tracking_area_code  =  1;

plmn_list = ({ mcc = 001; mnc = 01; mnc_length = 2; snssaiList = ({ sst = 1 }) });

nr_cellid = 12345678L;

////////// Physical parameters:

sib1_tda                                                  = 15;

min_rxtxtime = 6;

servingCellConfigCommon = (

{

#spCellConfigCommon

physCellId                                                    = 0;

#  downlinkConfigCommon

#frequencyInfoDL

# this is 3600 MHz + 12 PRBs@30kHz SCS (same as initial BWP)

absoluteFrequencySSB                                             = 640320;

dl_frequencyBand                                                 = 78;

# this is 3600 MHz

dl_absoluteFrequencyPointA                                       = 640000;

#scs-SpecificCarrierList

dl_offstToCarrier                                              = 0;

# subcarrierSpacing

# 0=kHz15, 1=kHz30, 2=kHz60, 3=kHz120

dl_subcarrierSpacing                                           = 1;

dl_carrierBandwidth                                            = 51;

#initialDownlinkBWP

#genericParameters

# this is RBstart=27,L=48 (275*(L-1))+RBstart

initialDLBWPlocationAndBandwidth                               =  12925;

# subcarrierSpacing

# 0=kHz15, 1=kHz30, 2=kHz60, 3=kHz120

initialDLBWPsubcarrierSpacing                                   = 1;

#pdcch-ConfigCommon

initialDLBWPcontrolResourceSetZero                              = 1;

initialDLBWPsearchSpaceZero                                     = 0;

#uplinkConfigCommon

#frequencyInfoUL

ul_frequencyBand                                              = 78;

#scs-SpecificCarrierList

ul_offstToCarrier                                             = 0;

# subcarrierSpacing

# 0=kHz15, 1=kHz30, 2=kHz60, 3=kHz120

ul_subcarrierSpacing                                          = 1;

ul_carrierBandwidth                                           = 51;

pMax                                                          = 20;

#initialUplinkBWP

#genericParameters

initialULBWPlocationAndBandwidth                            = 12925;

# subcarrierSpacing

# 0=kHz15, 1=kHz30, 2=kHz60, 3=kHz120

initialULBWPsubcarrierSpacing                               = 1;

#rach-ConfigCommon

#rach-ConfigGeneric

prach_ConfigurationIndex                                  = 98;

#prach_msg1_FDM

#0 = one, 1=two, 2=four, 3=eight

prach_msg1_FDM                                            = 0;

prach_msg1_FrequencyStart                                 = 0;

zeroCorrelationZoneConfig                                 = 13;

preambleReceivedTargetPower                               = -96;

#preamblTransMax (0...10) = (3,4,5,6,7,8,10,20,50,100,200)

preambleTransMax                                          = 6;

#powerRampingStep

# 0=dB0,1=dB2,2=dB4,3=dB6

powerRampingStep                                            = 1;

#ssb_perRACH_OccasionAndCB_PreamblesPerSSB_PR

#1=oneeighth,2=onefourth,3=half,4=one,5=two,6=four,7=eight,8=sixteen

ssb_perRACH_OccasionAndCB_PreamblesPerSSB_PR                = 4;

#one (0..15) 4,8,12,16,...60,64

ssb_perRACH_OccasionAndCB_PreamblesPerSSB                   = 14;

#ra_ContentionResolutionTimer

#(0..7) 8,16,24,32,40,48,56,64

ra_ContentionResolutionTimer                                = 7;

rsrp_ThresholdSSB                                           = 19;

#prach-RootSequenceIndex_PR

#1 = 839, 2 = 139

prach_RootSequenceIndex_PR                                  = 2;

prach_RootSequenceIndex                                     = 1;

# SCS for msg1, can only be 15 for 30 kHz < 6 GHz, takes precendence over the one derived from prach-ConfigIndex

#

msg1_SubcarrierSpacing                                      = 1,

# restrictedSetConfig

# 0=unrestricted, 1=restricted type A, 2=restricted type B

restrictedSetConfig                                         = 0,

msg3_DeltaPreamble                                          = 1;

p0_NominalWithGrant                                         =-90;

# pucch-ConfigCommon setup :

# pucchGroupHopping

# 0 = neither, 1= group hopping, 2=sequence hopping

pucchGroupHopping                                           = 0;

hoppingId                                                   = 40;

p0_nominal                                                  = -90;

ssb_PositionsInBurst_Bitmap                                   = 1;

# ssb_periodicityServingCell

# 0 = ms5, 1=ms10, 2=ms20, 3=ms40, 4=ms80, 5=ms160, 6=spare2, 7=spare1

ssb_periodicityServingCell                                    = 2;

# dmrs_TypeA_position

# 0 = pos2, 1 = pos3

dmrs_TypeA_Position                                           = 0;

# subcarrierSpacing

# 0=kHz15, 1=kHz30, 2=kHz60, 3=kHz120

subcarrierSpacing                                             = 1;

#tdd-UL-DL-ConfigurationCommon

# subcarrierSpacing

# 0=kHz15, 1=kHz30, 2=kHz60, 3=kHz120

referenceSubcarrierSpacing                                    = 1;

# pattern1

# dl_UL_TransmissionPeriodicity

# 0=ms0p5, 1=ms0p625, 2=ms1, 3=ms1p25, 4=ms2, 5=ms2p5, 6=ms5, 7=ms10

dl_UL_TransmissionPeriodicity                                 = 6;

nrofDownlinkSlots                                             = 7;

nrofDownlinkSymbols                                           = 6;

nrofUplinkSlots                                               = 2;

nrofUplinkSymbols                                             = 4;

ssPBCH_BlockPower                                             = -25;

}

);

# ------- SCTP definitions

SCTP :

{

# Number of streams to use in input/output

SCTP_INSTREAMS  = 2;

SCTP_OUTSTREAMS = 2;

};

////////// AMF parameters:

amf_ip_address = ({ ipv4 = "192.168.1.42"; });

NETWORK_INTERFACES :

{

GNB_IPV4_ADDRESS_FOR_NG_AMF              = "192.168.1.43/24";

GNB_IPV4_ADDRESS_FOR_NGU                 = "192.168.1.43/24";

GNB_PORT_FOR_S1U                         = 2152; # Spec 2152

};

}

);

MACRLCs = (

{

num_cc                      = 1;

tr_s_preference             = "local_L1";

tr_n_preference             = "local_RRC";

pusch_TargetSNRx10          = 150;

pucch_TargetSNRx10          = 200;

}

);

L1s = (

{

num_cc = 1;

tr_n_preference       = "local_mac";

pusch_proc_threads    = 8;

prach_dtx_threshold   = 120;

pucch0_dtx_threshold  = 150;

ofdm_offset_divisor   = 8; #set this to UINT_MAX for offset 0

}

);

RUs = (

{

local_rf       = "yes"

nb_tx          = 1

nb_rx          = 1

att_tx         = 0

att_rx         = 0;

bands          = [78];

max_pdschReferenceSignalPower = -27;

max_rxgain                    = 114;

sf_extension                  = 0;

eNB_instances  = [0];

clock_src = "internal";

}

);

rfsimulator :

{

serveraddr = "server";

serverport = 4043;

options = (); #("saviq"); or/and "chanmod"

modelname = "AWGN";

IQfile = "/tmp/rfsimulator.iqs";

};

security = {

# preferred ciphering algorithms

# the first one of the list that an UE supports in chosen

# valid values: nea0, nea1, nea2, nea3

ciphering_algorithms = ( "nea0" );

# preferred integrity algorithms

# the first one of the list that an UE supports in chosen

# valid values: nia0, nia1, nia2, nia3

integrity_algorithms = ( "nia2", "nia0" );

# setting 'drb_ciphering' to "no" disables ciphering for DRBs, no matter

# what 'ciphering_algorithms' configures; same thing for 'drb_integrity'

drb_ciphering = "yes";

drb_integrity = "no";

};

log_config :

{

global_log_level                      ="info";

global_log_verbosity                  ="medium";

hw_log_level                          ="info";

hw_log_verbosity                      ="medium";

phy_log_level                         ="info";

phy_log_verbosity                     ="medium";

mac_log_level                         ="info";

mac_log_verbosity                     ="high";

rlc_log_level                         ="info";

rlc_log_verbosity                     ="medium";

pdcp_log_level                        ="info";

pdcp_log_verbosity                    ="medium";

rrc_log_level                         ="info";

rrc_log_verbosity                     ="medium";

ngap_log_level                        ="debug";

ngap_log_verbosity                    ="medium";

};

---------------------------------------------------------------------------------------------------------------------------------------------

This is the UE command that I ran-----
sudo ./nr-uesoftmodem -O ~/openairinterface5g/targets/PROJECTS/GENERIC-NR-5GC/CONF/ue.conf --band 78 -C 3609180000 -r 51 --numerology 1 --ssb 40 --ue-fo-compensation

---------------------------------------------------------------------------------------------------------------------------------------------