-----------------------------------------------Steps to install the CN and gNB---------------------------------------------

->https://hackmd.io/@praveeng/6GMLAB-OAI-5G-SETUP-GUIDE#OAI-5G-SA-Setup-Guide

->https://gitlab.eurecom.fr/oai/openairinterface5g/-/blob/c5aeaf03dc9d50bc46a9955e97b0556261f359fd/doc/NR_SA_Tutorial_COTS_UE.md#32-build-oai-gnb

OS - ubuntu 22.04 LTS

Before even installing we must install the following pre-requisites-----

1) Low latency Kernel---

sudo apt update

sudo apt install -y linux-lowlatency

uname -r

output:- 5.15.0-*-lowlatency

2) Set CPU C-State and Disable P-State in GRUB

sudo nano /etc/default/grub

- in that change only the below lines from

GRUB_CMDLINE_LINUX_DEFAULT="quiet splash"

to -

GRUB_CMDLINE_LINUX_DEFAULT="quiet splash intel_pstate=disable processor.max_cstate=1 idle=poll nosoftlockup"

then ctrl+o -> Enter -> ctrl+x

then -

sudo update-grub

sudo reboot

to verify--

cat /proc/cmdline

3) Blacklist intel_powerclamp module

sudo bash -c 'echo "blacklist intel_powerclamp" >> /etc/modprobe.d/blacklist.conf'

sudo update-initramfs -u

sudo reboot

then verify if its not loaded , when enter the below command you should not get anything-

lsmod | grep intel_powerclamp

4) Disable Hyper-Threading

In BIOS - Hyper-Threading = Disabled

5) Disable CPU Frequency Scaling (Governor to “performance”)

for CPUFREQ in /sys/devices/system/cpu/cpu*/cpufreq/scaling_governor; do

echo performance | sudo tee $CPUFREQ

done

To make it persistent after reboot:----------

sudo apt install -y cpufrequtils

echo 'GOVERNOR="performance"' | sudo tee /etc/default/cpufrequtils

sudo systemctl enable cpufrequtils

6) Disable Power Management and Turbo in BIOS

in BIOS - Disable Intel SpeedStep (EIST)

Disable C-States (all)

Disable Turbo Mode

Set Power Management → Maximum Performance

7) Verify Your Real-Time System

cat /sys/devices/system/cpu/cpu*/cpufreq/scaling_governor

output-it should say PERFORMANCE

8) Install i7z utility to check the cpu-

sudo apt-get install i7z

sudo i7z

9) Install cpufrequtils:

sudo apt-get install cpufrequtils

sudo nano /etc/default/cpufrequtils

----then add this line-----GOVERNOR="performance"

then save and exit(ctrl+o -> Enter -> ctrl+x)

-----------------------------------------------OAI CN PC installation---------------------------------------------

1) Install Docker Engine and enable it--

sudo apt update

sudo apt install -y docker.io

sudo systemctl enable --now docker

verify-

docker --version

sudo systemctl status docker | grep Active

Expected: active (running)

2) Install Docker Compose plugin-

sudo apt install -y docker-compose

verify-

docker-compose version

Expected-.

version ≥ 1.29

3) Enable IP forwarding permanently-

# enable immediately

sudo sysctl -w net.ipv4.ip_forward=1

# make persistent

echo "net.ipv4.ip_forward=1" | sudo tee -a /etc/sysctl.conf

sudo sysctl -p

verify----

cat /proc/sys/net/ipv4/ip_forward

4) Quick re-check

docker --version

docker-compose version

cat /proc/sys/net/ipv4/ip_forward

----------------------------Installing OAI CN---------------------------

1) Clean start (optional but safe)

cd ~

sudo rm -rf oai-cn5g-fed

docker system prune -af

2) Clone the OAI CN5G repository

cd ~

git clone https://gitlab.eurecom.fr/oai/cn5g/oai-cn5g-fed.git

cd oai-cn5g-fed/docker-compose

3) Pull / fetch required Docker images

docker-compose -f docker-compose-basic-nrf.yaml pull

-------------then open oai-cn5g-fed/docker-compose/docker-compose-basic-nrf.yaml-------------(uncomment every CN Container's IP and do changes in oai-amf (already done below))

version: '3.8'

services:

mysql:

container_name: "mysql"

image: mysql:8.0

volumes:

- ./database/oai_db2.sql:/docker-entrypoint-initdb.d/oai_db.sql

- ./healthscripts/mysql-healthcheck2.sh:/tmp/mysql-healthcheck.sh

environment:

- TZ=Europe/Paris

- MYSQL_DATABASE=oai_db

- MYSQL_USER=test

- MYSQL_PASSWORD=test

- MYSQL_ROOT_PASSWORD=linux

healthcheck:

test: /bin/bash -c "/tmp/mysql-healthcheck.sh"

interval: 10s

timeout: 5s

retries: 30

networks:

public_net:

# For CI purposes, we are keeping the line commented

ipv4_address: 192.168.70.131

oai-udr:

container_name: "oai-udr"

image: oaisoftwarealliance/oai-udr:v2.1.0

expose:

- 80/tcp

- 8080/tcp

volumes:

- ./conf/basic_nrf_config.yaml:/openair-udr/etc/config.yaml

environment:

- TZ=Europe/Paris

depends_on:

- mysql

- oai-nrf

networks:

public_net:

# For CI purposes, we are keeping the line commented

ipv4_address: 192.168.70.136

oai-udm:

container_name: "oai-udm"

image: oaisoftwarealliance/oai-udm:v2.1.0

expose:

- 80/tcp

- 8080/tcp

volumes:

- ./conf/basic_nrf_config.yaml:/openair-udm/etc/config.yaml

environment:

- TZ=Europe/Paris

depends_on:

- oai-nrf

networks:

public_net:

# For CI purposes, we are keeping the line commented

ipv4_address: 192.168.70.137

oai-ausf:

container_name: "oai-ausf"

image: oaisoftwarealliance/oai-ausf:v2.1.0

expose:

- 80/tcp

- 8080/tcp

volumes:

- ./conf/basic_nrf_config.yaml:/openair-ausf/etc/config.yaml

environment:

- TZ=Europe/Paris

depends_on:

- oai-nrf

networks:

public_net:

# For CI purposes, we are keeping the line commented

ipv4_address: 192.168.70.138

oai-nrf:

container_name: "oai-nrf"

image: oaisoftwarealliance/oai-nrf:v2.1.0

expose:

- 80/tcp

- 8080/tcp

volumes:

- ./conf/basic_nrf_config.yaml:/openair-nrf/etc/config.yaml

environment:

- TZ=Europe/Paris

networks:

public_net:

# For CI purposes, we are keeping the line commented

ipv4_address: 192.168.70.130

oai-amf:

container_name: "oai-amf"

image: oaisoftwarealliance/oai-amf:v2.1.0

expose:

- 80/tcp

- 8080/tcp

- 38412/sctp

ports:

- "38412:38412/sctp"

volumes:

- ./conf/basic_nrf_config.yaml:/openair-amf/etc/config.yaml

environment:

- TZ=Europe/Paris

depends_on:

- oai-nrf

networks:

public_net:

# For CI purposes, we are keeping the line commented

ipv4_address: 192.168.70.132

oai-smf:

container_name: "oai-smf"

image: oaisoftwarealliance/oai-smf:v2.1.0

expose:

- 80/tcp

- 8080/tcp

- 8805/udp

volumes:

- ./conf/basic_nrf_config.yaml:/openair-smf/etc/config.yaml

environment:

- TZ=Europe/Paris

depends_on:

- oai-nrf

networks:

public_net:

# For CI purposes, we are keeping the line commented

ipv4_address: 192.168.70.133

oai-upf:

container_name: "oai-upf"

image: oaisoftwarealliance/oai-upf:v2.1.0

expose:

- 2152/udp

- 8805/udp

volumes:

- ./conf/basic_nrf_config.yaml:/openair-upf/etc/config.yaml

environment:

- TZ=Europe/Paris

depends_on:

- oai-nrf

cap_add:

- NET_ADMIN

- SYS_ADMIN

cap_drop:

- ALL

privileged: true

networks:

public_net:

# For CI purposes, we are keeping the line commented

ipv4_address: 192.168.70.134

oai-ext-dn:

privileged: true

init: true

container_name: oai-ext-dn

image: oaisoftwarealliance/trf-gen-cn5g:latest

environment:

- UPF_FQDN=oai-upf

- UE_NETWORK=12.1.1.0/24

- USE_FQDN=yes

healthcheck:

test: /bin/bash -c "ip r | grep 12.1.1"

interval: 10s

timeout: 5s

retries: 5

networks:

public_net:

# For CI purposes, we are keeping the line commented

ipv4_address: 192.168.70.135

networks:

public_net:

driver: bridge

name: demo-oai-public-net

ipam:

config:

- subnet: 192.168.70.128/26

driver_opts:

com.docker.network.bridge.name: "demo-oai"

-----------------------------open oai-cn5g-fed/docker-compose/conf

################################################################################

# Licensed to the OpenAirInterface (OAI) Software Alliance under one or more

# contributor license agreements.  See the NOTICE file distributed with

# this work for additional information regarding copyright ownership.

# The OpenAirInterface Software Alliance licenses this file to You under

# the OAI Public License, Version 1.1  (the "License"); you may not use this file

# except in compliance with the License.

# You may obtain a copy of the License at

#

#      http://www.openairinterface.org/?page_id=698

#

# Unless required by applicable law or agreed to in writing, software

# distributed under the License is distributed on an "AS IS" BASIS,

# WITHOUT WARRANTIES OR CONDITIONS OF ANY KIND, either express or implied.

# See the License for the specific language governing permissions and

# limitations under the License.

#-------------------------------------------------------------------------------

# For more information about the OpenAirInterface (OAI) Software Alliance:

#      contact@openairinterface.org

################################################################################

# OAI CN Configuration File

### This file can be used by all OAI NFs

### Some fields are specific to an NF and will be ignored by other NFs

## NOTE ON YAML ANCHORS ##

# We use YAML anchors to ease the configuration and to avoid duplicating parts of the configuration.

# This is especially true for the SNSSAIs, as we have to define them for multiple NFs.

# Please note that the use of anchors is not mandatory, and you can define the SNSSAI in each NF yourself.

# You can read more about anchors here: https://yaml.org/spec/1.2.2/#anchors-and-aliases

############# Common configuration

# Log level for all the NFs

log_level:

general: debug

# If you enable registration, the other NFs will use the NRF discovery mechanism

register_nf:

general: yes

http_version: 2

############## SBI Interfaces

### Each NF takes its local SBI interfaces and remote interfaces from here, unless it gets them using NRF discovery mechanisms

nfs:

amf:

host: oai-amf

sbi:

port: 8080

api_version: v1

interface_name: eth0

n2:

interface_name: eth0

port: 38412

smf:

host: oai-smf

sbi:

port: 8080

api_version: v1

interface_name: eth0

n4:

interface_name: eth0

port: 8805

upf:

host: oai-upf

sbi:

port: 8080

api_version: v1

interface_name: eth0

n3:

interface_name: eth0

port: 2152

n4:

interface_name: eth0

port: 8805

n6:

interface_name: eth0

n9:

interface_name: eth0

port: 2152

udm:

host: oai-udm

sbi:

port: 8080

api_version: v1

interface_name: eth0

udr:

host: oai-udr

sbi:

port: 8080

api_version: v1

interface_name: eth0

ausf:

host: oai-ausf

sbi:

port: 8080

api_version: v1

interface_name: eth0

nrf:

host: oai-nrf

sbi:

port: 8080

api_version: v1

interface_name: eth0

#### Common for UDR and AMF

database:

host: mysql

user: test

type: mysql

password: test

database_name: oai_db

generate_random: true

connection_timeout: 300 # seconds

## general single_nssai configuration

## Defines YAML anchors, which are reused in the config file

snssais:

- &embb_slice1

sst: 1

- &embb_slice2

sst: 1

sd: 000001 # in hex

- &custom_slice

sst: 222

sd: 00007B # in hex

############## NF-specific configuration

amf:

amf_name: "OAI-AMF"

# This really depends on if we want to keep the "mini" version or not

support_features_options:

enable_simple_scenario: no # "no" by default with the normal deployment scenarios with AMF/SMF/UPF/AUSF/UDM/UDR/NRF.

# set it to "yes" to use with the minimalist deployment scenario (including only AMF/SMF/UPF) by using the internal AUSF/UDM implemented inside AMF.

# There's no NRF in this scenario, SMF info is taken from "nfs" section.

enable_nssf: no

enable_smf_selection: yes

relative_capacity: 30

statistics_timer_interval: 20  # in seconds

emergency_support: false

served_guami_list:

- mcc: 001                         # changed by me

mnc: 01				# changed by me

amf_region_id: 01

amf_set_id: 001

amf_pointer: 01

- mcc: 001				# changed by me

mnc: 01				# changed by me

amf_region_id: 01

amf_set_id: 001

amf_pointer: 01

plmn_support_list:

- mcc: 001

mnc: 01

tac: 1				# changed by me

nssai:

- *embb_slice1

- *embb_slice2

- *custom_slice

supported_integrity_algorithms:

- "NIA0"

- "NIA1"

- "NIA2"

supported_encryption_algorithms:

- "NEA0"

- "NEA1"

- "NEA2"

smf:

ue_mtu: 1500

support_features:

use_local_subscription_info: yes # Use infos from local_subscription_info or from UDM

use_local_pcc_rules: yes # Use infos from local_pcc_rules or from PCF

# we resolve from NRF, this is just to configure usage_reporting

upfs:

- host: oai-upf

config:

enable_usage_reporting: no

ue_dns:

primary_ipv4: "172.21.3.100"

primary_ipv6: "2001:4860:4860::8888"

secondary_ipv4: "8.8.8.8"

secondary_ipv6: "2001:4860:4860::8888"

ims:

pcscf_ipv4: "127.0.0.1"

pcscf_ipv6: "fe80::7915:f408:1787:db8b"

# the DNN you configure here should be configured in "dnns"

# follows the SmfInfo datatype from 3GPP TS 29.510

smf_info:

sNssaiSmfInfoList:

- sNssai: *embb_slice1

dnnSmfInfoList:

- dnn: "oai"

- sNssai: *embb_slice2

dnnSmfInfoList:

- dnn: "oai.ipv4"

- sNssai: *custom_slice

dnnSmfInfoList:

- dnn: "default"

local_subscription_infos:

- single_nssai: *embb_slice1

dnn: "oai"

qos_profile:

5qi: 9

session_ambr_ul: "200Mbps"

session_ambr_dl: "400Mbps"

- single_nssai: *embb_slice2

dnn: "oai.ipv4"

qos_profile:

5qi: 9

session_ambr_ul: "100Mbps"

session_ambr_dl: "200Mbps"

- single_nssai: *custom_slice

dnn: "default"

qos_profile:

5qi: 9

session_ambr_ul: "50Mbps"

session_ambr_dl: "100Mbps"

upf:

support_features:

enable_bpf_datapath: no    # If "on": BPF is used as datapath else simpleswitch is used, DEFAULT= off

enable_snat: yes           # If "on": Source natting is done for UE, DEFAULT= off

remote_n6_gw: localhost      # Dummy host since simple-switch does not use N6 GW

smfs:

- host: oai-smf            # To be used for PFCP association in case of no-NRF

upf_info:

sNssaiUpfInfoList:

- sNssai: *embb_slice1

dnnUpfInfoList:

- dnn: "oai"

- sNssai: *embb_slice2

dnnUpfInfoList:

- dnn: "oai.ipv4"

- sNssai: *custom_slice

dnnUpfInfoList:

- dnn: "default"

## DNN configuration

dnns:

- dnn: "oai"

pdu_session_type: "IPV4"

ipv4_subnet: "12.1.1.128/25"

- dnn: "oai.ipv4"

pdu_session_type: "IPV4"

ipv4_subnet: "12.1.1.64/26"

- dnn: "default"

pdu_session_type: "IPV4"

ipv4_subnet: "12.1.1.0/26"

- dnn: "ims"

pdu_session_type: "IPV4V6"

ipv4_subnet: "14.1.1.2/24"

---------------------------------------------------------how to run the CN------------------------------------

1) Start CN (simulation mode)

cd ~/oai-cn5g-fed/docker-compose

python3 core-network.py --type start-basic

2) Verify containers are running and healthy

docker ps --format "table {{.Names}}\t{{.Status}}\t{{.Ports}}"

Expected-

oai-nrf     Up (healthy)

oai-amf     Up (healthy)

oai-smf     Up (healthy)

oai-upf     Up (healthy)

oai-ausf    Up (healthy)

oai-udm     Up (healthy)

oai-udr     Up (healthy)

mysql       Up (healthy)

3) Check NRF Registration-

sudo apt install -y jq

curl -s -X GET --http2-prior-knowledge "http://192.168.70.130:8080/nnrf-nfm/v1/nf-instances?nf-type=NRF" | jq .

curl -s -X GET --http2-prior-knowledge "http://192.168.70.130:8080/nnrf-nfm/v1/nf-instances?nf-type=AMF" | jq .

curl -s -X GET --http2-prior-knowledge "http://192.168.70.130:8080/nnrf-nfm/v1/nf-instances?nf-type=SMF" | jq .

curl -s -X GET --http2-prior-knowledge "http://192.168.70.130:8080/nnrf-nfm/v1/nf-instances?nf-type=UPF" | jq .

curl -s -X GET --http2-prior-knowledge "http://192.168.70.130:8080/nnrf-nfm/v1/nf-instances?nf-type=UDM" | jq .

curl -s -X GET --http2-prior-knowledge "http://192.168.70.130:8080/nnrf-nfm/v1/nf-instances?nf-type=AUSF" | jq .

expected-----------------------

dtri@dtri-H510M-H:~/oai-cn5g-fed/docker-compose$ curl -s -X GET --http2-prior-knowledge "http://192.168.70.130:8080/nnrf-nfm/v1/nf-instances?nf-type=NRF" | jq .

{

"_links": {

"item": [],

"self": ""

}

}

dtri@dtri-H510M-H:~/oai-cn5g-fed/docker-compose$ curl -s -X GET --http2-prior-knowledge "http://192.168.70.130:8080/nnrf-nfm/v1/nf-instances?nf-type=AMF" | jq .

curl -s -X GET --http2-prior-knowledge "http://192.168.70.130:8080/nnrf-nfm/v1/nf-instances?nf-type=SMF" | jq .

curl -s -X GET --http2-prior-knowledge "http://192.168.70.130:8080/nnrf-nfm/v1/nf-instances?nf-type=UPF" | jq .

curl -s -X GET --http2-prior-knowledge "http://192.168.70.130:8080/nnrf-nfm/v1/nf-instances?nf-type=UDM" | jq .

curl -s -X GET --http2-prior-knowledge "http://192.168.70.130:8080/nnrf-nfm/v1/nf-instances?nf-type=AUSF" | jq .

{

"_links": {

"item": [

{

"href": "192.168.70.132"

}

],

"self": ""

}

}

{

"_links": {

"item": [

{

"href": "192.168.70.133"

}

],

"self": ""

}

}

{

"_links": {

"item": [

{

"href": "192.168.70.134"

}

],

"self": ""

}

}

{

"_links": {

"item": [

{

"href": "192.168.70.137"

}

],

"self": ""

}

}

{

"_links": {

"item": [

{

"href": "192.168.70.138"

}

],

"self": ""

}

}

4) Verify CN Readiness for gNB Connection

docker exec -it oai-amf bash

netstat -an | grep 38412

expected----

sctp      LISTEN     0      5      0.0.0.0:38412

5) Then after running the gNB we should see in the CN PC that the gNB is connected-----

-docker logs oai-amf -f

the above command starts the heartbeat which refreshes and there in the table we must make sure that the gNB is visible and it shows connected

Pictures Just for reference----

If the SCTP Link is established but not connected then it will show as below----
this happens because of some configuration error

If the SCTP Link is only not established then u will get the table as blank
sometimes there wont be any error displaying but the table will be empty that means that the CN and gNB didn’t even establish the connection in the first place then the configuration might be not correct or incorrect IP configuration