------------------------------------------------setting up the OAI gNB in PC----------------------------

follow the same steps of pre-requisites as the CN as both require the exact same steps

also i have attached the 2-links where i had referred and then only start installing

---------------------------------------------------------------------------------------------------------

1) nstall OAI gNB (Clean)

sudo rm -rf ~/openairinterface5g

cd ~

git clone https://gitlab.eurecom.fr/oai/openairinterface5g.git

cd openairinterface5g

git checkout develop

source oaienv

cd cmake_targets

./build_oai -I

./build_oai -w USRP --gNB

after that check all the configuration files are present or not----

~/openairinterface5g/targets/PROJECTS/GENERIC-NR-5GC/CONF$ ls channelmod_rfsimu_LEO_satellite.conf gnb-pnf.band78.rfsim.2x2.conf gnb.sa.band77.273prb.fhi72.4x4-vvdn.conf gnb.sa.band78.fr1.106PRB.usrpb210.sabox.conf cu_gnb.conf gnb-pnf.band78.rfsim.conf gnb.sa.band77.fr1.273PRB.2x2.usrpn300.conf gnb.sa.band78.fr1.106PRB.usrpb210.yaml du_gnb.conf gnb.sa.band1.u0.52PRB.usrpb210.conf gnb.sa.band77.fr1.273PRB.usrpx300.conf gnb.sa.band78.fr1.162PRB.2x2.usrpn300.conf gnb0.prs.band261.fr2.64PRB.usrpx310.conf gnb.sa.band254.ntn.mu1.24prb.rfsim.conf gnb.sa.band78.106PRB.usrpb210.RA_2-Step.conf gnb.sa.band78.fr1.189PRB.rfsim.conf gnb0.prs.band78.fr1.106PRB.usrpx310.conf gnb.sa.band255.ntn.mu0.25prb.rfsim.conf gnb.sa.band78.273prb.fhi72.4x2-benetel550.conf gnb.sa.band78.fr1.217PRB.2x2.usrpn300.conf gnb1.prs.band261.fr2.64PRB.usrpx310.conf gnb.sa.band256.ntn.mu0.25prb.rfsim.conf gnb.sa.band78.273prb.fhi72.4x4-benetel550.conf gnb.sa.band78.fr1.24PRB.usrpb210.conf gnb1.prs.band78.fr1.106PRB.usrpx310.conf gnb.sa.band257.u3.32prb.usrpx410.conf gnb.sa.band78.273prb.fhi72.4x4-benetel550-mplane.conf gnb-vnf.sa.band78.106prb.nfapi.conf gnb.band78.sa.fr1.106PRB.2x2.usrpn310.conf gnb.sa.band41.fr1.106PRB.usrpb210.conf gnb.sa.band78.273prb.fhi72.4x4-foxconn.conf gnb-vnf.sa.band78.273prb.aerial.conf gnb.band78.sa.fr1.162PRB.2x2.usrpn310.conf gnb.sa.band41.fr1.52PRB.usrpb210.conf gnb.sa.band78.273prb.fhi72.4x4-liteon.conf gnb-vnf.sa.band78.273prb.nfapi.conf gnb-cu.sa.f1.conf gnb.sa.band66.fr1.106PRB.usrpn300.conf gnb.sa.band78.273prb.fhi72.4x4-metanoia.conf gnb-vnf.sa.cbrs.aerial.conf gnb-du.sa.band66.25prb.rfsim.pci0.conf gnb.sa.band66.fr1.106PRB.usrpx300.conf gnb.sa.band78.fr1.106PRB.2x2.usrpn300.conf neighbour-config-rfsim.conf gnb-du.sa.band66.25prb.rfsim.pci1.conf gnb.sa.band66.fr1.24PRB.usrpx300.conf gnb.sa.band78.fr1.106PRB.pci0.rfsim.conf uecap_ports1.xml gnb-du.sa.band77.273prb.fhi72.4x4-benetel650.conf gnb.sa.band66.fr1.25PRB.usrpx300.conf gnb.sa.band78.fr1.106PRB.pci1.rfsim.conf uecap_ports2.xml gnb-du.sa.band77.273prb.fhi72.8x8-benetel650_650.conf gnb.sa.band66.u0.25prb.rfsim.conf gnb.sa.band78.fr1.106PRB.usrpb210.2pattern.conf uecap_ports4.xml gnb-du.sa.band77.273prb.fhi72.8x8-benetel650_650-mplane.conf gnb.sa.band77.106prb.fhi72.4x4-vvdn.conf gnb.sa.band78.fr1.106PRB.usrpb210.4layer.conf ue.conf gnb-du.sa.band78.106prb.rfsim.pci0.conf gnb.sa.band77.162prb.usrpn310.4x4.conf gnb.sa.band78.fr1.106PRB.usrpb210.conf ue.nr.prs.fr1.106prb.conf gnb-du.sa.band78.106prb.rfsim.pci1.conf gnb.sa.band77.273prb.fhi72.2x2-vvdn.conf gnb.sa.band78.fr1.106PRB.usrpb210.redcap.yaml ue.nr.prs.fr2.64prb.conf

-----------------here i have used gnb.sa.band78.fr1.106PRB.pci0.rfsim.conf file becuse this helps in simple rfsimulation--------------------

Active_gNBs = ( "gNB-OAI");

# Asn1_verbosity, choice in: none, info, annoying

Asn1_verbosity = "none";

gNBs =

(

{

////////// Identification parameters:

gNB_ID    =  0xe00;

gNB_name  =  "gNB-OAI";

// Tracking area code, 0x0000 and 0xfffe are reserved values

tracking_area_code  =  1;

plmn_list = ({ mcc = 001; mnc = 01; mnc_length = 2; snssaiList = ({ sst = 1; }) });

@include "neighbour-config-rfsim.conf"

nr_cellid = 12345678L;

////////// Physical parameters:

do_CSIRS                                                  = 1;

do_SRS                                                    = 1;

#uess_agg_levels = [0,1,2,2,1]

servingCellConfigCommon = (

{

#spCellConfigCommon

physCellId                                                    = 0;

#  downlinkConfigCommon

#frequencyInfoDL

# this is 3600 MHz + 43 PRBs@30kHz SCS (same as initial BWP)

absoluteFrequencySSB                                             = 641280;

dl_frequencyBand                                                 = 78;

# this is 3600 MHz

dl_absoluteFrequencyPointA                                       = 640008;

#scs-SpecificCarrierList

dl_offstToCarrier                                              = 0;

# subcarrierSpacing

# 0=kHz15, 1=kHz30, 2=kHz60, 3=kHz120

dl_subcarrierSpacing                                           = 1;

dl_carrierBandwidth                                            = 106;

#initialDownlinkBWP

#genericParameters

# this is RBstart=27,L=48 (275*(L-1))+RBstart

initialDLBWPlocationAndBandwidth                               = 28875; # 6366 12925 12956 28875 12952

# subcarrierSpacing

# 0=kHz15, 1=kHz30, 2=kHz60, 3=kHz120

initialDLBWPsubcarrierSpacing                                   = 1;

#pdcch-ConfigCommon

initialDLBWPcontrolResourceSetZero                              = 12;

initialDLBWPsearchSpaceZero                                     = 0;

#uplinkConfigCommon

#frequencyInfoUL

ul_frequencyBand                                              = 78;

#scs-SpecificCarrierList

ul_offstToCarrier                                             = 0;

# subcarrierSpacing

# 0=kHz15, 1=kHz30, 2=kHz60, 3=kHz120

ul_subcarrierSpacing                                          = 1;

ul_carrierBandwidth                                           = 106;

pMax                                                          = 20;

#initialUplinkBWP

#genericParameters

initialULBWPlocationAndBandwidth                            = 28875;

# subcarrierSpacing

# 0=kHz15, 1=kHz30, 2=kHz60, 3=kHz120

initialULBWPsubcarrierSpacing                               = 1;

#rach-ConfigCommon

#rach-ConfigGeneric

prach_ConfigurationIndex                                  = 98;

#prach_msg1_FDM

#0 = one, 1=two, 2=four, 3=eight

prach_msg1_FDM                                            = 0;

prach_msg1_FrequencyStart                                 = 0;

zeroCorrelationZoneConfig                                 = 13;

preambleReceivedTargetPower                               = -96;

#preamblTransMax (0...10) = (3,4,5,6,7,8,10,20,50,100,200)

preambleTransMax                                          = 6;

#powerRampingStep

# 0=dB0,1=dB2,2=dB4,3=dB6

powerRampingStep                                            = 1;

#ssb_perRACH_OccasionAndCB_PreamblesPerSSB_PR

#1=oneeighth,2=onefourth,3=half,4=one,5=two,6=four,7=eight,8=sixteen

ssb_perRACH_OccasionAndCB_PreamblesPerSSB_PR                = 4;

#one (0..15) 4,8,12,16,...60,64

ssb_perRACH_OccasionAndCB_PreamblesPerSSB                   = 14;

#ra_ContentionResolutionTimer

#(0..7) 8,16,24,32,40,48,56,64

ra_ContentionResolutionTimer                                = 7;

rsrp_ThresholdSSB                                           = 19;

#prach-RootSequenceIndex_PR

#1 = 839, 2 = 139

prach_RootSequenceIndex_PR                                  = 2;

prach_RootSequenceIndex                                     = 1;

# SCS for msg1, can only be 15 for 30 kHz < 6 GHz, takes precendence over the one derived from prach-ConfigIndex

#

msg1_SubcarrierSpacing                                      = 1,

# restrictedSetConfig

# 0=unrestricted, 1=restricted type A, 2=restricted type B

restrictedSetConfig                                         = 0,

msg3_DeltaPreamble                                          = 1;

p0_NominalWithGrant                                         =-90;

# pucch-ConfigCommon setup :

# pucchGroupHopping

# 0 = neither, 1= group hopping, 2=sequence hopping

pucchGroupHopping                                           = 0;

hoppingId                                                   = 40;

p0_nominal                                                  = -90;

ssb_PositionsInBurst_Bitmap                                   = 1;

# ssb_periodicityServingCell

# 0 = ms5, 1=ms10, 2=ms20, 3=ms40, 4=ms80, 5=ms160, 6=spare2, 7=spare1

ssb_periodicityServingCell                                    = 2;

# dmrs_TypeA_position

# 0 = pos2, 1 = pos3

dmrs_TypeA_Position                                           = 0;

# subcarrierSpacing

# 0=kHz15, 1=kHz30, 2=kHz60, 3=kHz120

subcarrierSpacing                                             = 1;

#tdd-UL-DL-ConfigurationCommon

# subcarrierSpacing

# 0=kHz15, 1=kHz30, 2=kHz60, 3=kHz120

referenceSubcarrierSpacing                                    = 1;

# pattern1

# dl_UL_TransmissionPeriodicity

# 0=ms0p5, 1=ms0p625, 2=ms1, 3=ms1p25, 4=ms2, 5=ms2p5, 6=ms5, 7=ms10

dl_UL_TransmissionPeriodicity                                 = 6;

nrofDownlinkSlots                                             = 7;

nrofDownlinkSymbols                                           = 6;

nrofUplinkSlots                                               = 2;

nrofUplinkSymbols                                             = 4;

ssPBCH_BlockPower                                             = -25;

}

);

# ------- SCTP definitions

SCTP :

{

# Number of streams to use in input/output

SCTP_INSTREAMS  = 2;

SCTP_OUTSTREAMS = 2;

};

////////// AMF parameters:

amf_ip_address = ({ ipv4 = "192.168.1.43"; });                                                  -----------> CN PC IP

NETWORK_INTERFACES :

{

GNB_IPV4_ADDRESS_FOR_NG_AMF              = "192.168.1.53/24";				    ------------> gNB PC IP

GNB_IPV4_ADDRESS_FOR_NGU                 = "192.168.1.53/24";				    ------------> gNB PC IP

GNB_PORT_FOR_S1U                         = 2152; # Spec 2152

};

}

);

MACRLCs = (

{

num_cc                      = 1;

tr_s_preference             = "local_L1";

tr_n_preference             = "local_RRC";

pusch_TargetSNRx10          = 150;

pucch_TargetSNRx10          = 200;

}

);

L1s = (

{

num_cc = 1;

tr_n_preference       = "local_mac";

prach_dtx_threshold   = 120;

pucch0_dtx_threshold  = 100;

ofdm_offset_divisor   = 8; #set this to UINT_MAX for offset 0

}

);

RUs = (

{

local_rf       = "yes"

nb_tx          = 1

nb_rx          = 1

att_tx         = 12;

att_rx         = 12;

bands          = [78];

max_pdschReferenceSignalPower = -27;

max_rxgain                    = 114;

eNB_instances  = [0];

clock_src = "internal";

}

);

rfsimulator :

{

serveraddr = "server";

serverport = 4043;

options = (); #("saviq"); or/and "chanmod"

modelname = "AWGN";

IQfile = "/tmp/rfsimulator.iqs";

};

security = {

# preferred ciphering algorithms

# the first one of the list that an UE supports in chosen

# valid values: nea0, nea1, nea2, nea3

ciphering_algorithms = ( "nea0" );

# preferred integrity algorithms

# the first one of the list that an UE supports in chosen

# valid values: nia0, nia1, nia2, nia3

integrity_algorithms = ( "nia2", "nia0" );

# setting 'drb_ciphering' to "no" disables ciphering for DRBs, no matter

# what 'ciphering_algorithms' configures; same thing for 'drb_integrity'

drb_ciphering = "yes";

drb_integrity = "no";

};

log_config :

{

global_log_level                      ="info";

hw_log_level                          ="info";

phy_log_level                         ="info";

mac_log_level                         ="info";

rlc_log_level                         ="info";

pdcp_log_level                        ="info";

rrc_log_level                         ="info";

ngap_log_level                        ="debug";

f1ap_log_level                        ="debug";

};

e2_agent = {

near_ric_ip_addr = "127.0.0.1";

#sm_dir = "/path/where/the/SMs/are/located/"

sm_dir = "/usr/local/lib/flexric/"

};

--------------------------------------------------------------------------------------------------------

after pasting this ------

First always try ping in both the systems that is the CN and the gNB PC

in gNB PC -ping CN_IP (eg:- ping 192.168.1.43 to check IP of your PC type the command --ip addr show)

in CN PC - ping gNB_IP

then if everything is working then-

-first run the CN PC and open the heartbeat of CN in Terminal then after opening it

-then in gNB PC run this command -

sudo -E ./nr-softmodem --rfsim   -O ../../../targets/PROJECTS/GENERIC-NR-5GC/CONF/gnb.sa.band78.fr1.106PRB.pci0.rfsim.conf

this is what we will get if there are no errors and the gNB successfully connects to the CN---