-> Setup & Install Dependencies

sudo apt-get install build-essential g++ libpcsclite-dev pcscd pcsc-tools

-> Download & Compile the Programming Tool

cd ~

wget https://open-cells.com/d5138782a8739209ec5760865b1e53b0/uicc-v3.3.tgz

tar -xvf uicc-v3.3.tgz

cd ~/uicc-v3.3

rm program_uicc

make

->Connect the SIM Programmer

ls /dev/ttyUSB*

=>should see :- /dev/ttyUSB0

->Set permissions:

sudo chmod 666 /dev/ttyUSB0

-> Verify SIM is Readable:

cd ~/uicc-v3.3

sudo ./program_uicc --port /dev/ttyUSB0

=>should see :- IMSI, ICCID, SPN printed.

-> Program the SIM:-

=> if UE parameter is : -

uicc0 = {

imsi = "001010000000001";

key  = "0C0A34601D4F07677303652C0462535B";

opc  = "63bfa50ee6523365ff14c1f45f88737d";

dnn  = "oai";

nssai_sst = 1;

nssai_sd  = 1;

}

position0 = {

x = 0.0;

y = 0.0;

z = 6377900.0;

}

Then enter the command in terminal :-

sudo ./program_uicc \ --port /dev/ttyUSB0 \ --adm 12345678\ --imsi 001010000000001 \ --isdn 00000001 \ --acc 0001 \ --key 0C0A34601D4F07677303652C0462535B \ --opc 63bfa50ee6523365ff14c1f45f88737d \ --spn "OAI-Lab"\ --authenticate \ --noreadafter

-> Verify SIM Programming:-

sudo ./program_uicc --port /dev/ttyUSB0

-> Register UE in Core Network

1)

REPLACE INTO AuthenticationSubscription (

ueid, authenticationMethod, encPermanentKey, encOpcKey,

protectionParameterId, sequenceNumber, authenticationManagementField, algorithmId

) VALUES (

'001010000000001', '5G_AKA',

'0C0A34601D4F07677303652C0462535B',

'63bfa50ee6523365ff14c1f45f88737d',

'1',

JSON_OBJECT('sequenceNumber','000000000080'),

'8000', '5G_AKA'

);

2)

REPLACE INTO AccessAndMobilitySubscriptionData (

ueid, servingPlmnid, subscribedUeAmbr, nssai, subsRegTimer

) VALUES (

'001010000000001', '00101',

JSON_OBJECT('uplink','100 Mbps','downlink','100 Mbps'),

JSON_OBJECT('defaultSingleNssais',JSON_ARRAY(JSON_OBJECT('sst',1,'sd','000001'))),

600

);

3)

REPLACE INTO SessionManagementSubscriptionData (

ueid, servingPlmnid, singleNssai, dnnConfigurations

) VALUES (

'001010000000001', '00101',

JSON_OBJECT('sst',1,'sd','000001'),

JSON_OBJECT('oai',JSON_OBJECT(

'sscModes',JSON_OBJECT('defaultSscMode','SSC_MODE_1','allowedSscModes',JSON_ARRAY('SSC_MODE_1')),

'pduSessionTypes',JSON_OBJECT('defaultSessionType','IPV4','allowedSessionTypes',JSON_ARRAY('IPV4')),

'5gQosProfile',JSON_OBJECT('5qi',9,'arp',JSON_OBJECT('priorityLevel',8)),

'sessionAmbr',JSON_OBJECT('uplink','100 Mbps','downlink','100 Mbps')

))

);

4)

REPLACE INTO SmfSelectionSubscriptionData (

ueid, servingPlmnid

) VALUES (

'001010000000001', '00101'

);

-> Verify :-

1) SELECT * FROM AuthenticationSubscription WHERE ueid='001010000000001'\G

2) SELECT * FROM AccessAndMobilitySubscriptionData WHERE ueid='001010000000001'\G

3) SELECT * FROM SessionManagementSubscriptionData WHERE ueid='001010000000001'\G

4) SELECT * FROM SmfSelectionSubscriptionData WHERE ueid='001010000000001'\G